# PROCEDURE

- Take the SEM image of the microstructure in .pgm format.

Fig. Grayscale micrograph

- Use the /img_to_ell/threshold tool to convert the .pgm image into a black & white .pgm image containing the inclusions.

Fig. B&W micrograph

- Use an image editor to open the black & white image from Step 2 and save it as a B&W (dithered) .pbm image. The despeckle algorithm of the image editor can be used to smooth the edges of the particles.

- Use the /img_to_ell/img-to-ell tool to analyze the .pbm image and produce a random1.dat file, which describes the inclusions in a simple format to be used by the mesh generator.

Fig. Inclusions plot from random1.dat file

- To plot the inclusions described in the random1.dat file, use the MATLB program /img_to_ell/plotell/plot_ell_dat.m

- Use /meshgen/mesh to read the random1.dat file and create a preliminary VCFEM mesh in the file vchomo.inp.

Fig. Plot of vchomo.inp mesh file.

- This vchomo.inp file is used as a input for the /meshgen/add_node tool to add more nodes in the mesh. The new output file is finalmesh.inp
Note 1: Make any relevant changes to the geometry of the mesh in vchomo.inp before using add_node to create the final mesh.

Note 2: The source code in meshgen/add_node.f90 can be changed to output custom material properties and to control the number of nodes added to the preliminary mesh.

- The file /meshgen/plot_mesh.m is a MATLAB script that can be used to plot both the vchomo.inp and the finalmesh.inp VCFEM mesh.

- Copy the finalmesh.inp file to the simulation directory and run the simulation.

- The EQV_CURVE.dat file generated by the simulation has the equivalent stress-strain curve.

- Use /postproc/extract/extract_m2 to extract the data from the results_un.dat file generated by the simulation to create two new files: matrix.dat and inclusion.dat.

Note: For executing the extract_m2 tool, use /postproc/extract/extract_m2

- Use /postproc/contour/ to create contour plots from matrix.dat and inclustion.dat
Usage: vtkpython CMRL.py
Note: Both the matrix.dat file and the inclusion.dat file must be inside the “/postproc/contour/” folder.